IS23SC1604

Features

! 16K bits (2K x 8) serial EEPROM with security logic

!	Independent Security Codes for Data access after validation operations

!	Provides transportation code security

!	Manufactured with low power CMOS technology

!	Single 5V supply with Vpp internally generated

!	2µs Read access time; 5mS Write/Erase time

!	Operating temperature range from 0C to 70C

!	Highly reliable 1,000,000 erase/write cycles; 10 years data retention

Security

! Data access only after validation of Security Codes

! Independent Read/Write protection of memory zones

! Permanent invalidation of Security Codes after eight false attempts

! Two security levels for Card Issuer and End User

! Permanent fuse protection on passwords and personal data

Description

IS23SC1604 is a low-cost and low power, highly

secured 16K bits (2Kx8) serial EEPROM. It is fabricated using ISSI’s advanced CMOS technology.

The IS23SC1604 provides 15704 bits of memory in four application zones. Each application zone is protected by multiple security codes independently from unauthorized read/write/erase access. An internal security fuse protects the personalized data written by card issuer.

Integrated Silicon Solution, Inc.

2231 Lawson Lane  °   Santa Clara, CA 95054   °   Tel: 408.588.0800  °   Fax: 408.588.0806   °   Email: sales@issi.com   °   Web: www.issi.com

To Order

ISSI REGIONAL SALES OFFICES Headquarters & West Coast Office Santa Clara, California

PHONE: 	408-588-0800

FAX: 	408-588-0805

E-MAIL:    sales@issi.com

Nashua, NH

PHONE: 	603-594-4176

FAX: 	603-594-4181

E-MAIL:    gpolanec@msn.com

| SIO contact PAD number | name | Description |
| --- | --- | --- |
| C1 | Vcc | 5V power Supply |

Dallas, TX

PHONE: 	972-488-9691

E-MAIL:    lzastrow@flash.net

Devon, England

PHONE: 	44-1752-764285

E-MAIL:    glouch@issi.com05

China Offices

Shanghai

PHONE: 	+86 21 5080 2288

FAX: 	+86 21 5080 2028

E-MAIL:    issish@sh163c.sta.net.cn

Shenzhen

PHONE: 	86755-663-3977

FAX: 	86755-663-3984

E-MAIL:    issi@public.szptt.net.cn

Suzhou

PHONE: 	86512-725-0384

FAX: 	86512-725-0536

E-MAIL:    issisz@public1.sz.js.cn

Beijing

PHONE: 	86-10-6824-3301

FAX: 	86-10-6816-9097

E-MAIL:    issibj@public.bta.net.cn

Hong Kong

PHONE: 	852-2319-2211

FAX: 	852-2319-2004

C2	RST	Reset

C3	CLK	Serial Clock and Address Control

C4	FUS	Security fuse Pin

C5	GND	Ground

C6	N/C	No connec

C7	I/O	Bi-direction data input/output

C8	PGM	Programming control for(write/erase)

IS23SC1604

Integrated Silicon Solution, Inc.

2231 Lawson Lane  °  Santa Clara, CA 95054  °  Tel: 408.588.0800  °  Fax: 408.588.0806  °  Email: sales@issi.com

Web: www.issi.com