Security & Chip Card ICs

SLE 44C20S

8-bit Security Controller with

17-Kbyte ROM, 256 byte RAM

2-Kbyte EEPROM and Sleep Mode

Short Product Information 10.01

| SLE 44C20S Short Product Information Ref.: SPI_SLE 44C20SU_1001 |  |
| --- | --- |
| This document contains preliminary information on a new product under development. Details are subject to change without notice. Revision History: Current Version 10.01 |  |
| Previous Releases: 07.99 |  |
| Page |  |

Important:   Further information is confidential and on request. Please contact: Infineon Technologies AG in Munich, Germany,

Security & Chip Card ICs, Tel  +49 - (0)89 234-80000

Fax +49 - (0)89 234-81000

E-Mail: security.chipcard.ics@infineon.com

Edition 2001

Published by Infineon Technologies AG, CC Applications Group

St.-Martin-Strasse 53, D-81541 München

© Infineon Technologies AG 2001

All Rights Reserved.

Attention please!

The information herein is given to describe certain components and shall not be considered as warranted characteristics. Terms of delivery and rights to technical change reserved.

We hereby disclaim any and all warranties, including but not limited to warranties of non-infringement, regarding circuits, descriptions and charts stated herein.

Infineon Technologies is an approved CECC manufacturer.

Information

For  further  information  on  technology,  delivery  terms  and  conditions  and  prices  please  contact  your  nearest  Infineon

Technologies Office in Germany or our Infineon Technologies Representatives world-wide (see address list).

Warnings

Due to technical requirements components may contain dangerous substances. For information on the types in question please contact your nearest Infineon Technologies Office.

Infineon Technologies Components may only be used in life-support devices or systems with the express written approval of Infineon Technologies, if a failure of such components can reasonably be expected to cause the failure of that life-support device or system, or to affect the safety or effectiveness of that device or system. Life support devices or systems are intended to be implanted in the human body, or to support and/or maintain and sustain and/or protect human life. If they fail, it is reasonable to assume that the health of the user or other persons may be endangered.

8-bit Security Controller with

17-Kbyte ROM, 256-byte RAM, 2-Kbyte EEPROM and Sleep Mode

Features

8-bit microcomputer in CMOS technology Instruction   set  opcode   compatible   with standard SAB8051 processor

Dedicated, non-standard architecture with execution time less than half of standard SAB 8051 processor

15-Kbyte 	User 	ROM 	for 	application programs

2-Kbyte 	manufacturer 	ROM 	for 	Chip

Management System (CMS)

2-Kbyte 	EEPROM 	as 	program/data memory

256-byte RAM

Power saving sleep mode

Clock freq. = int. freq.:

1 to 5 MHz1) at 5 V 	10 %,

1 to 4 MHz at 3 V 	10 %

Contact  configuration  and serial interface in accordance with ISO 7816

Supply voltage range: 2.7 V to 5.5 V

< 10 mA supply current at 5 MHz Temperature range: – 25 to + 70 °C. 2) ESD protection larger than 4 kV

Document References

Confidential Data Book SLE 44CxxS Instruction Set SLE44CxxS – Quick

Reference

Qualification report

Chip delivery  specification  for wafer  with chip-layout (die size, orientation,...)

Module specification containing description of package, etc.

Qualification report module

Development Tools Overview

Short 	Product 	Information 	Software

Development Kit SDK CC

EEPROM

Reading, erasing and writing byte by byte Flexible   page   mode   for   1   to   8   bytes write/erase operation

32 bytes security area

Write time 3.5 ms, erase time 1.75 ms Frequency-adaptable programming time Minimum of 500,000 write/erase cycles3) Data retention for minimum of ten years.

EEPROM  programming  voltage  generated on chip

Security Features

ROM code not visible due to implantation

Low voltage sensor High voltage sensor Low-frequency sensor

High-frequency protection

16 	bytes 	security 	PROM, 	hardware protected

Unique  chip identification  number  for each chip

CMS

Intelligent  write/erase  routines  for  N  bytes programming (0 < N < 256)

Two serial interface modes according to ISO

7816-3:

– 	9600 bit/s related to 3.57 MHz

– 	9600 bit/s related to 4.91 MHz

Support

HW-& SW-Tools (Emulator, Card Emulator, Simulator)

Application notes

Supported Standards

ISO/IEC 7816

EMV 2000

GSM 11.1x

ETS I  TS 102 221

1)    Extended frequency range up to 7.5 MHz is available, see ordering information.

2)    Extended temperature range is available for certain applications, e.g. GSM, see ordering information.

3)    Values are temperature dependent for further information please refer to your Infineon Technologies

Sales Office.

Ordering Information

| Type | Package1 | Voltage Range | Temperature Range | Frequency Range |
| --- | --- | --- | --- | --- |
| SLE 44C20S-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz @ 5 V 1 MHz – 4 MHz @ 3V |
| SLE 44C20S -C | C |  |  |  |
| SLE 44C20S -S | S |  |  |  |
| SLE 44C20S -T85-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz @ 5 V 1 MHz – 4 MHz @ 3V |
| SLE 44C20S -T85-C | C |  |  |  |
| SLE 44C20S -T85-S | S |  |  |  |
| SLE 44C20S -V5-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz |
| SLE 44C20S -V5-C | C |  |  |  |
| SLE 44C20S -V5-S | S |  |  |  |
| SLE 44C20S -V5-T85-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz |
| SLE 44C20S -V5-T85-C | C |  |  |  |
| SLE 44C20S -V5-T85-S | S |  |  |  |
| SLE 44C20S -V5-F7-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 7.5 MHz |
| SLE 44C20S -V5-F7-C | C |  |  |  |
| SLE 44C20S -V5-F7-S | S |  |  |  |

Production sites: Regensburg SLE 44CxxS UMC Taiwan SLE 44CxxU

Pin Description

1 available as wire-bonded module (M4) for embedding in plastic cards, as die (C) for customer packaging or on request as SMT package (S)

Figure 1 	Pin Configuration (top view)

Pin Definitions and Functions

| Card Contact | Symbol | Function |
| --- | --- | --- |
| C1 | VCC | Operating voltage |
| C2 | RST | Reset input |
| C3 | CLK | Processor clock input |
| C5 | GND | Ground |
| C4;C6,C8 | N.C. | Not connected |
| C7 | I/O | Bi-directional data port |

General Description

SLE 44C20S is a member of the Infineon Technologies 44 security microcontroller series, especially designed for smart card applications. The device is fabricated in a  Infineon Technologies proprietary CMOS technology, resulting in a significant reduction of die size. New features such as low voltage operation, extended page mode and  I/O routines offer additional performance required in applications like 3V SIM cards for GSM, payment, health care, loyalty or electronic purses.