Security & Chip Card ICs

SLE 44C42S

8-bit Security Controller with

17-Kbyte ROM, 256 byte RAM

4-Kbyte EEPROM and Sleep Mode

Short Product Information 07.99

| SLE 44C42S Short Product Information |  |
| --- | --- |
| Revision History: Current Version 07.99 |  |
| Previous Releases: 2.0 (06.98) |  |
| Page | Subjects (changes since last revision) |
|  | Layout change |

Important:Further information is confidential and on request. Please contact: Infineon Technologies AG in Munich, Germany,

Security & Chip Card ICs, Fax +49  89 234-28925

Published by Infineon Technologies AG i.Gr., CC Applications Group

St.-Martin-Strasse, D-81541 München

© Infineon Technologies AG i.Gr. 1999

All Rights Reserved.

Attention please!

The  information  herein  is  given  to  describe  certain  components  and  shall  not  be  considered  as  warranted characteristics.

Terms of delivery and rights to technical change reserved.

We hereby disclaim any and all warranties, including but not limited to warranties of non-infringement, regarding circuits, descriptions and charts stated herein.

Infineon Technologies is an approved CECC manufacturer.

Information

For further information on technology, delivery terms and conditions and prices please contact your nearest Infineon

Technologies Office in Germany or our Infineon Technologies Representatives world-wide (see address list).

Warnings

Due to technical requirements components may contain dangerous substances. For information on  the  types  in question please contact your nearest Infineon Technologies Office.

Infineon Technologies Components may only be used in life-support devices or systems with the express written approval of Infineon Technologies, if a failure of such components can reasonably be expected to cause the failure of that life-support device or system, or to affect the safety or effectiveness of that device or system. Life support devices or systems are intended to be implanted in the human body, or to support and/or maintain and sustain and/or protect human life. If they fail, it is reasonable to assume that the health of the user or other persons may be endangered.

8-bit Security Controller with

17-Kbyte ROM, 256-byte RAM, 4-Kbyte EEPROM and Sleep Mode

Features

8-bit microcomputer in CMOS

technology

Instruction set opcode compatible with standard SAB8051 processor

Software compatible with SLE 44C40

Dedicated, non-standard architecture with execution time less than half of standard SAB 8051 processor

15-Kbyte User ROM for application programs

2-Kbyte manufacturer ROM for Chip

Management System (CMS)

4-Kbyte EEPROM as program/data memory

256-byte RAM

Power saving sleep mode

Clock freq. = int. freq.:

1 to 5 MHz1) at 5 V	10 %,

1 to 4 MHz at 3 V	10 %

Contact configuration and serial interface in accordance with ISO7816

Supply voltage range: 2.7 V to 5.5 V

< 10 mA supply current at 5 MHz Temperature range: – 25 to + 70 °C 2) ESD protection larger than 4 kV

EEPROM

Reading, erasing and writing byte by byte

Flexible page mode for 1 to 16 bytes write/erase operation

32 bytes security area

Write time 3.5 ms, erase time 1.75 ms

Frequency-adaptable programming time

Minimum of 500,000 write/erase cycles3)

Data retention for minimum of ten years

EEPROM programming voltage generated on chip

Security Features

ROM code not visible due to implantation

Low voltage sensor High voltage sensor Low-frequency sensor

High-frequency protection

16 bytes security PROM, hardware protected

Unique chip identification number for each chip

CMS

Intelligent write/erase routines for N

bytes programming (0 < N < 256)

Two serial interface modes according to ISO 7816-3:

– 9600 bit/s related to 3.57 MHz

– 9600 bit/s related to 4.91 MHz

1)    Extended frequency range up to 7.5 MHz is available, see ordering information.

2)    Extended temperature range is available for certain applications, e.g. GSM, see ordering information.

3)    Values are temperature dependent, for further information please refer to your Infineon Technologies

Sales Office.

Ordering Information

| Type | Package1 | Voltage Range | Temperature Range | Frequency Range |
| --- | --- | --- | --- | --- |
| SLE 44C42S-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz @ 5V 1 MHz – 4 MHz @ 3V |
| SLE 44C42S -C | C |  |  |  |
| SLE 44C42S -T85-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz @ 5V 1 MHz – 4 MHz @ 3V |
| SLE 44C42S -T85-C | C |  |  |  |
| SLE 44C42S -V5-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz |
| SLE 44C42S -V5-C | C |  |  |  |
| SLE 44C42S -V5-T85-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz |
| SLE 44C42S -V5-T85-C | C |  |  |  |
| SLE 44C42S -V5-F7-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 7.5 MHz |
| SLE 44C42S -V5-F7-C | C |  |  |  |

Pin Description

Figure 1	Pin Configuration (top view)

1 available as wire-bonded module (M4) for embedding in plastic cards or as die (C) for customer packaging

Pin Definitions and Functions

| Card Contact | Symbol | Function |
| --- | --- | --- |
| C1 | VCC | Operating voltage |
| C2 | RST | Reset input |
| C3 | CLK | Processor clock input |
| C5 | GND | Ground |
| C4;C6,C8 | N.C. | Not connected |
| C7 | I/O | Bi-directional data port |

General Description

SLE 44C42S is a member of the Infineon Technologies 44 security microcontroller family, especially designed for smart card applications. The devices fabricated in an Infineon Technologies proprietary CMOS technology, resulting in a significant reduction of die size compared to the SLE 44C40. New features such as low voltage operation, extended page mode and I/O routines offer additional performance required in applications like 3V SIM cards for GSM, banking, health care, security access or loyalty, while maintaining software compatibility to the SLE 44C40.