Security & Chip Card ICs

SLE 44C80S

8-bit Security Controller with

17-Kbyte ROM, 256 byte RAM

8 Kbyte EEPROM and Sleep Mode

Short Product Information 10.01

| SLE 44C80S Short Product Information Ref.: SPI_SLE 44C80SU_1001 |  |
| --- | --- |
| This document contains preliminary information on a new product under development. Details are subject to change without notice. Revision History: Current Version 10.01 |  |
| Previous Releases: 07.99 |  |
| Page |  |

Important:   Further information is confidential and on request. Please contact: Infineon Technologies AG in Munich, Germany,

Security & Chip Card ICs, Tel  +49 - (0)89 234-80000

Fax +49 - (0)89 234-81000

E-Mail: security.chipcard.ics@infineon.com

Edition 2001

Published by Infineon Technologies AG, CC Applications Group

St.-Martin-Strasse 53, D-81541 München

© Infineon Technologies AG 2001

All Rights Reserved.

Attention please!

The information herein is given to describe certain components and shall not be considered as warranted characteristics. Terms of delivery and rights to technical change reserved.

We hereby disclaim any and all warranties, including but not limited to warranties of non-infringement, regarding circuits, descriptions and charts stated herein.

Infineon Technologies is an approved CECC manufacturer.

Information

For  further  information  on  technology,  delivery  terms  and  conditions  and  prices  please  contact  your  nearest  Infineon

Technologies Office in Germany or our Infineon Technologies Representatives world-wide (see address list).

Warnings

Due to technical requirements components may contain dangerous substances. For information on the types in question please contact your nearest Infineon Technologies Office.

Infineon Technologies Components may only be used in life-support devices or systems with the express written approval of Infineon Technologies, if a failure of such components can reasonably be expected to cause the failure of that life-support device or system, or to affect the safety or effectiveness of that device or system. Life support devices or systems are intended to be implanted in the human body, or to support and/or maintain and sustain and/or protect human life. If they fail, it is reasonable to assume that the health of the user or other persons may be endangered.

8-bit Security Controller with

17-Kbyte ROM, 256-byte RAM, 8-Kbyte EEPROM and Sleep Mode

Features

8-bit microcomputer in CMOS technology Instruction set opcode compatible with standard SAB8051 processor

Software compatible with SLE 44C80

Dedicated, non-standard architecture with execution time less than half of standard SAB 8051 processor

15-Kbyte User ROM for application programs

2-Kbyte manufacturer ROM for Chip

Management System (CMS)

8-Kbyte EEPROM as program/data memory

256-byte RAM

Power saving sleep mode

Clock freq. = int. freq.:

1 to 5 MHz1) at 5 V 	10 %,

1 to 4 MHz at 3 V 	10 %

Contact configuration and serial interface in accordance with ISO7816

Supply voltage range: 2.7 V to 5.5 V

< 10 mA supply current at 5 MHz Temperature range: – 25 to + 70 °C 2) ESD protection larger than 4 kV

Document References

Confidential Data Book SLE 44CxxS Instruction Set SLE44CxxS – Quick

Reference

Qualification report

Chip delivery specification for wafer with chip-layout (die size, orientation,...)

Module specification containing description of package, etc.

Qualification report module

Development Tools Overview

Short 	Product 	Information 	Software

Development Kit SDK CC

EEPROM

Reading, erasing and writing byte by byte Flexible page mode for 1 to 32 bytes write/erase operation

32 bytes security area

Write time 3.5 ms, erase time 1.75 ms Frequency-adaptable programming time Minimum of 500,000 write/erase cycles 3) Data retention for minimum of ten years EEPROM programming voltage generated on chip

Security Features

ROM code not visible due to implantation

Low voltage sensor High voltage sensor Low-frequency sensor

High-frequency protection

16 bytes security PROM, hardware protected

Unique chip identification number for each chip

CMS

Intelligent write/erase routines for N bytes programming (0 < N < 256)

Two serial interface modes according to

ISO 7816-3:

– 	9600 bit/s related to 3.57 MHz

– 	9600 bit/s related to 4.91 MHz

Support

HW-& SW-Tools (Emulator, Card Emulator, Simulator)

Application notes

Supported Standards

ISO/IEC 7816

EMV 2000

GSM 11.1x

ETS I  TS 102 221

1)    Extended frequency range up to 7.5 MHz is available, see ordering information.

2)    Extended temperature range is available for certain applications, e.g. GSM, see ordering information

3)    Values are temperature dependent for further information please refer to your Infineon Technologies

Sales Officer.

Ordering Information

| Type | Package1 | Voltage Range | Temperature Range | Frequency Range |
| --- | --- | --- | --- | --- |
| SLE 44C80S-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz @ 5V 1 MHz – 4 MHz @ 3V |
| SLE 44C80S -C | C |  |  |  |
| SLE 44C80S -T85-M4 | M4 | 2.7 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz @ 5V 1 MHz – 4 MHz @ 3V |
| SLE 44C80S -T85-C | C |  |  |  |
| SLE 44C80S -V5-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 5 MHz |
| SLE 44C80S -V5-C | C |  |  |  |
| SLE 44C80S -V5-T85-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 85°C | 1 MHz - 5 MHz |
| SLE 44C80S -V5-T85-C | C |  |  |  |
| SLE 44C80S -V5-F7-M4 | M4 | 4.5 V - 5.5 V | – 25°C to + 70°C | 1 MHz - 7.5 MHz |
| SLE 44C80S -V5-F7-C | C |  |  |  |

Production sites:

Regensburg SLE 44CxxS UMC Taiwan SLE 44CxxU

1 available as wire-bonded module (M4) for embedding in plastic cards or as die (C) for customer packaging

Pin Description

Figure 1 	Pin Configuration (top view)

Pin Definitions and Functions

| Card Contact | Symbol | Function |
| --- | --- | --- |
| C1 | VCC | Operating voltage |
| C2 | RST | Reset input |
| C3 | CLK | Processor clock input |
| C5 | GND | Ground |
| C4;C6,C8 | N.C. | Not connected |
| C7 | I/O | Bi-directional data port |

General Description

SLE 44C80S is a member of the Infineon Technologies 44 security microcontroller family, especially designed for smart card applications. The device is fabricated in an  Infineon Technologies proprietary CMOS technology, resulting in a significant reduction of die size compared to the SLE 44C80S. New features such as low voltage operation, extended page mode and I/O routines offer additional performance required in applications like 3V SIM  cards for GSM, payment,  pay-TV  or  security  access  while  maintaining  software compatibility to the SLE 44C80S manufactured in the previous technology.